# Temporary Access Regulation
and Protection Against Vehicle Overruns

SAFETY TECHNOLOGY

Installation and Setup Manual
for Mobile Steel Road Barrier

IronBloX.de

# Introduction

## Purpose

This installation and setup manual is intended to ensure the safe and proper use and deployment of a mobile road barrier. It is designed to ensure that all employees correctly set up, operate and remove the barrier in order to maintain traffic safety and prevent accidents. Compliance with these instructions is mandatory to ensure smooth operations and the safety of everyone involved.

The barrier is intended to prevent vehicles from entering restricted areas, for example by driving between two barriers positioned next to each other or by overcoming the barriers by force.

## Scope of Delivery

| Item | Quantity per unit |
| --- | --- |
| X-element (steel, welded) | 2 |
| M20 threaded rod (hot-dip galvanized) | 1 |
| Washers DIN 125 / M20 | 2 |
| Rubber feet (optional) | As required |
| Film cover | 1 |
| Chain cover | 1 (between each 2 barriers) |
| Connecting chain (steel, galvanized, optional) | 2 (between each 2 barriers) |
| Securing element (e.g. cotter pin, locking washer) | 1 set |
| M20 nut (self-locking or with locknut) | 2 |

# Required Tools

Ring spanner or socket (AF 30 / AF 32, depending on nut size)

Torque wrench (recommended: 120–150 Nm)

Pliers or cotter-pin punch, if required

PPE: protective gloves, safety footwear, high-visibility vest if required

# Safety and Application Instructions

Assembly may only be carried out by instructed personnel.

Do not set up in the traffic area without reflective material / lighting.

Select a level, load-bearing surface (asphalt, concrete, paving).

Remove dirt, leaves or ice from the surface.

Determine the installation direction: the front side of the “X” shape faces the direction of travel.

# Installation Instructions for the Road Barrier

## 1. Site Preparation

Prepare and check the designated installation area before assembly.

# 2. Positioning the X-elements

Insert two X-elements into each other at 90°.

Align the openings for the central threaded rod with each other.

# 3. Inserting the Threaded Rod

Insert the threaded rod from one side through the holes of both X-elements.

Fit one washer and one nut on each side.

# 4. Tensioning the Structure

Tighten both nuts evenly until the X-elements are firmly seated against each other.

Tighten using a torque wrench (recommended: at least 120 Nm).

Secure the connection with a cotter pin or locknut.

# 5. Checking Stability

Set up the barrier in the previously marked areas.

The barrier must not wobble or rattle.

Simulate lateral loading (light pressure test using body weight).

Optionally connect two barriers with chains.

Place the film cover over the barrier (openings facing the chain side).

# 6. Connecting Road Barriers with Chains (Optional)

Locate the integrated eyelets (on the side or front of the X-element).

Route the connecting chain (e.g. 12 mm link chain, hot-dip galvanized) between two barriers.

Secure with carabiners or shackles.

Check the tension: the chain should be guided along the ground.

Fit the chains with an anti-overflow cover (chain cover).

Optional: secure with a padlock (e.g. for official/public-authority deployments).

# Disassembly and Storage

Loosen the nuts evenly on both sides.

Store the X-elements flat (stackable).

Place chains in loops and, if necessary, spray with corrosion protection.

Store threaded rods and nuts in a separate container.

# Optional Additions

Checklist for setup teams

Maintenance schedule (including visual inspection for cracks, corrosion and material deformation)

Instruction sheet for personnel in accordance with DGUV

# Checklist for Setup Teams

Mobile Road Barrier

Deployment location: ______________________________

Date: ____________________    Site supervisor: ____________________

## Before Setup

☐ Surface is level and load-bearing

☐ X-elements correctly positioned

☐ PPE available (footwear, gloves, high-visibility vest if required)

☐ Completeness of all components

☐ Visual inspection of components (no rust, no deformation)

☐ Tools complete and suitable

## During Setup

☐ Threaded rod centered and passed through both elements

☐ Washers and nuts installed

☐ Torque check (120–150 Nm)

☐ Cotter pin/locknut fitted for securing

☐ Chain (if required) tensioned and secured

☐ Film cover correctly positioned

# After Setup

☐ Stability checked

☐ Barrier stands securely (no wobbling)

☐ Visible warning devices installed (e.g. reflectors)

☐ Film cover fitted over the barrier

## Manual Review / Deviations

The installation and setup manual does not require any adjustments.

The following points of the installation and setup manual require adjustment to the actual conditions:

____________________________________________________________________________

____________________________________________________________________________

## Confirmation of Signatures

The undersigned persons hereby confirm that the above-mentioned points have been carried out properly:

Site supervisor: __________________________    Signature: __________________________

Team member: _____________________________    Signature: __________________________

Date: ____________________                  Date: ____________________

Site supervisor's note regarding the setup and any identified and avoidable risks:

____________________________________________________________________________

____________________________________________________________________________

# Maintenance Schedule

Mobile Road Barrier

User: ____________________    Date: ____________________    Site supervisor: ____________________

| Maintenance item | Interval | Procedure / Inspection criterion |
| --- | --- | --- |
| Visual inspection for corrosion | Before every use | Remove rust spots; repaint if necessary |
| Inspection of connecting components | Before every use | Check threads, nuts, washers and cotter pin for secure fit |
| Chain cleaning | Monthly | Degrease; lubricate again if necessary |
| Replacement in case of damage | Immediately | Cracks or deformation → take out of service immediately |
| Functional check of fastening | Every 6 months | Test assembly including torque check |
| Film cover | Regularly | Check for damage; replace if necessary |

## Confirmation of Signatures

The undersigned persons hereby confirm that the above-mentioned points have been carried out properly:

Site supervisor: __________________________    Signature: __________________________

Date: ____________________

# Instruction Sheet

Operation & Safety (DGUV)

User: ____________________    Date: ____________________    Site supervisor: ____________________

## Purpose of the Instruction

Safe deployment and handling of mobile road barriers in order to prevent accidents and hazards to persons and traffic.

## Instruction Content

Rules of conduct and safety measures

Legal requirements and permits

Selection of the deployment location and environmental assessment

Setup and securing of the barrier

Use of personal protective equipment (e.g. high-visibility vests, protective helmets)

Safety measures during deployment

Conduct in case of specific hazards (e.g. darkness, poor visibility)

Dismantling and follow-up after deployment

Emergency measures and communication

## Mandatory Rules

Carry out deployment only after prior instruction.

Always wear personal protective equipment.

Maintain a safe distance.

Ensure visibility through high-visibility vests and reflective elements.

If unsure, stop the deployment immediately and inform the supervisor.

Leave the deployment site clean and undamaged.

# Notes and Special Features

## Safety Rules

Observe local conditions and traffic regulations.

Use additional lighting in darkness.

In the event of accidents or incidents, immediately call the emergency services.

Assembly may only be carried out by trained personnel.

PPE is mandatory.

Observe the operating instructions.

Never set up on sloping or unsecured ground.

Always secure chain connections where crowds of people are present.

In an emergency, keep the setup quick to dismantle.

## Assembly Instructions

The threaded rod must pass through both X-elements.

Tighten the nuts evenly against each other.

Securing with a cotter pin or locknut is required.

## Confirmation of Instruction

I hereby confirm that I have taken note of and understood the above-mentioned contents concerning the safe handling of mobile road barriers.

Name of person instructed: ______________________________

Signature of person instructed: __________________________

Signature of instructor: _________________________________